# Steps for fixing clothes

This step includes all steps for the 4 stations, which students first need to learn, in order to teach this to other students. Give them time to study these steps:

A — Fixing a button (5–8 steps)

- Thread needle (use doubled thread for strength) and knot the end.

- From the garment’s underside, push needle up through one hole of the button.

- Go down through the opposite hole and repeat 4–6 times (or 3–4 for kids).

- For 4-hole buttons: make an X or two parallel stitches.

- Optional: to give space (shank) for thick fabric, place a toothpick over button while stitching and remove it before knotting.

- Finish with 2 small backstitches on the underside, tie a secure knot and trim thread.

B — Closing a small hole (simple running / whipstitch)

- Trim loose threads and place cloth over a darning egg/cardboard to stretch fabric flat.

- For tiny holes: use running stitch across the hole to pull threads together.

- For larger holes: place a small patch behind the hole and use whipstitch around the edge to secure.

- Tie off underneath and check from front for neatness.

C — Sewing on a patch

- Cut patch ~1–2 cm larger than hole. Pin patch in place (under or over the hole).

- Use a running or blanket stitch around the edge, keeping stitches even (about 3–5 mm).

- For fast options, use iron-on patches but an adult must handle the iron; reinforce with a few stitches.

D — Simple embroidery (outline/backstitch + lazy daisy)

- Transfer a small, simple shape (heart/flower) onto fabric. Hoop fabric taut.

- Start with a backstitch for outlines (good for names/letters).

- For petals, teach lazy daisy: loop stitch anchored with a small tack stitch.

- Finish by knotting thread at the back and trimming.